CSC1043 – Computer Networks 2

# Lab 3 – Configuration of Network and End Devices

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Router 1 | Fa0/0 | 192.168.100.1 | 255.255.255.0 |
|  | Fa1/0 | 192.168.30.1 | 255.255.255.252 |
| Router 2 | Fa0/0 | 10.0.0.1 | 255.0.0.0 |
|  | Fa1/0 | 192.168.30.2 | 255.255.255.252 |
| PC1 | Fa0 | 192.168.100.10 | 255.255.255.0 |
| PC2 | Fa0 | 10.0.0.5 | 255.255.255.0 |

Objectives

- Configure IP addresses of network and end devices

- Configure hostnames on the Cisco Internetwork Operating System (IOS) routers and switches using the command-line interface (CLI).

- Use Cisco IOS commands to specify or limit access to the device configurations.

- Use IOS commands to save the running configuration.

- Verify connectivity between the two PC end devices.

# Scenario

As a recently hired LAN technician, your network manager has asked you to demonstrate your ability to configure small networks. Your tasks include configuring initial settings on network devices using the Cisco IOS and configuring IP address parameters on host devices to provide end-to-end connectivity. Use the Packet Tracer Tutorial in Lab 1 to

# Instructions

Open the Lab3.pka file. When the User Profile window appears, enter your full name and email address exactly as registered on Loop. Failure to provide this information will result in a 0 grade for this lab assignment.

# Part 1: Configure IP addresses

## Step 1: Configure IP addresses of Router 1 and Router 2

Use the addressing table to configure the IP addresses and network mask of both routers. Make sure that the lights on the various cables turn green.

## Step 2: Configure IP addresses PC1 and PC2

Use the addressing table to configure the IP addresses, network mask, and default gateway of both PCs.

Part 2: Configure routing tables of Router 1 and Router 2

## Step 1: Configure the routing table of Router 1

- Click on Router 1 and select Static under the Routing heading.

- Enter the network IP address that PC2 belongs to, the subnet mask for that network, and the Next Hop, i.e., the IP address of the router through which packets should be forwarded to reach that network.

- Click on the add button.

## Step 2: Configure the routing table of Router 2

# Repeat the same steps as in Step 1, but this time configure the route to the network that PC1 belongs to.

Part 3: Verify connectivity between PC1 and PC2

- Open the Desktop tab on the PC1 and select the command prompt application.

- Ping PC2’s IP address.

- If everything is configured correctly, you should receive replies from PC2. If not, there is an issue. Switch to Simulation mode and repeat the ping command to identify where the problem occurs.

Part 4: Configure Router 1

## Step 1: Use console connection to connect PC1 to Router 1

Step 2: Basic configuration of Router 1. Refer to the Packet Tracer tutorial from Lab 1 for guidance.

- Click on PC1  Desktop  Terminal.

- Click OK without changing any of the Terminal Configuration parameters.

- Press Enter.

- Use the appropriate command to name Router 1 as R1.

- Use the appropriate command to set csc1042 as the password for the console line.

- Use the appropriate command to encrypt the password.

- Use the appropriate command to configure the following as the message-of-the-day (MOTD) banner: Warning: Authorized Users Only! Unauthorized use of this device is prohibited.

- Use the appropriate command to save your configuration.

- Type exit. You will return to the initial screen you saw when accessing Router 1 from PC1. Press Enter; if everything has been configured correctly, the device name should now appear as R1, and the banner message will be displayed.

Part 5: Configure Router 2

Repeat the same steps in Part 4. Connect PC2 to Router 2 to perform the configuration. Name Router 2 as R2.

Note: Click Check Results to see your progress. Click Reset Activity to generate a new set of requirements. If you click on this before you complete the activity, all configurations will be lost.