CSC1042 – Computer Networks 2

# Lab 2 - Connect devices and configure their IP addresses

# Addressing Table

| Device | Interface | IP Address | Connects To |
| --- | --- | --- | --- |
| Router 3 | FastEthernet1/0 | 192.168.30.1/24 | FastEthernet0/0 |
|  | FastEthernet0/0 | 192.168.5.1/24 | Fast Ethernet0/1 |
| Router 1 | Console | N/A | RS 232 |
|  | FastEthernet1/0 | 10.0.0.1/8 | FastEthernet0 |
|  | Serial 2/0 | 172.31.0.1/16 | Serial 3/0 |
| Router 2 | Serial 3/0 | 172.31.0.2/16 | Serial 2/0 |
|  | FastEthernet1/0 | 172.16.0.1/16 | FastEtherent0/1 |
|  | FastEthernet0/0 | 192.168.30.2/24 | FastEthernet1/0 |
| Home PC | FastEthernet0 | 192.168.5.100/24 | FastEthernet0/2 |
| Family PC | FastEthernet0 | 192.168.5.102/24 | FastEthernet0/3 |
| Web server | FastEthernet0 | 10.0.0.254/8 | FastEthernet0/1 |
| Configuration PC | RS 232 | N/A | Console |

# Background

When working in Packet Tracer, you should know how to select the appropriate cable and how to properly connect and configure devices. This activity will examine device configurations in Packet Tracer, selecting the proper cable based on the configuration, connecting the devices, and assigning them IP addresses.

# Instructions

Open the Lab2.pka file. When the User Profile window appears, enter your full name and email address exactly as registered on Loop. Failure to provide this information will result in a 0 grade for this lab assignment.

# Part 1: Connect Router 1

## Step 1: Connect Router 1 to Router 2.

Choose the correct cable to connect Router 1 Serial2/0 to Router2 Serial3/0. Use one of the available Serial

cables.

If you attached the correct cable, the link lights on the cable turn green.

## Step 2: Connect Router 1 to Web server.

Choose the correct cable to connect Router 1 FastEthernet1/0 to Web server FastEthernet0. Routers and computers traditionally use the same wires to transmit (1 and 2) and receive (3 and 6). The correct cable to choose consists of these crossed wires. Although many NICs can now autosense which pair is used to transmit and receive, Router 1 and Web server do not have autosensing NICs.

If you attached the correct cable, the link lights on the cable turn green.

## Step 3: Connect Router 1 to the Configuration PC.

Choose the correct cable to connect Router 1 Console to Configuration PC RS 232. This cable does not provide network access to Configuration PC but allows you to configure Router 1 through its terminal.

If you attached the correct cable, the link lights on the cable turn black.

# Part 3: Connect Remaining Devices

## Step 1: Connect Router 2 to Switch 1.

Choose the correct cable to connect Router 2 FastEthernet1/0 to Switch FastEthernet0/1.

If you attached the correct cable, the link lights on the cable turn green. Allow a few seconds for the light to transition from amber to green.

## Step 2: Connect Router 2 to Router 3.

Choose the correct cable to connect Router 2 FastEthernet0/0 to Router 3 FastEthernet1/0. If you attached the correct cable, the link lights on the cable will turn green.

## Step 3: Connect Router 3 to Switch 2.

Choose the correct cable to connect Router 3 FastEthernet0/0 to Switch 2 FastEthernet0/1. If you attached the correct cable, the link lights on the cable turn green.

## Step 4: Connect remaining devices to Switch 2.

Choose the correct cable to connect Switch 2 FastEthernet0/2 to Home PC FastEthernet0. If you attached the correct cable, the link lights on the cable turn green.

Choose the correct cable to connect Switch 2 FastEthernet0/3 to Family PC FastEthernet0. If you attached the correct cable, the link lights on the cable turn green.

# Part 4: IP addresses assignment

## Step 1: Assign IP address to the interfaces of Router 1 as shown in the Addressing Table.

- Click on Router 1 and select the Config Tab from the top menu.

- Configure Router 1’s interfaces based on the information in the Addressing Table.

## Step 2: Repeat the same process for Routers 2 and 3

## Step 3: Assign IP address to Web server

- Click on Web Server and select the Desktop Tab from the top menu.

- Click on the IP configuration application.

- Configure Web server IP address based on the information in the Addressing Table. You should fill in the IP address, the subnet mask, and the default gateway

## Step 4: Repeat the same process for Home PC and Family PC

# Part 5: Verify Connections

## Step 1: Test the connection between Family PC and Web Server.

- Open the Desktop tab on the Family PC and select the command prompt application.

- Ping Web server’s IP address.

- Open the Web Browser and type in the web address www.example.com. You might need to wait for 30 to 60 minutes before the web page is displayed.

## Step 2: Open Router 1 from Configuration PC.

- Open the Terminal application of Configuration Terminal and accept the default settings.

- Press Enter to view the Router 1 command prompt.

- Type show ip interface brief to view interface statuses.