CSC1042 – Computer Networks 2

# Lab 4 - Design and Implement a VLSM Addressing Scheme

# Background / Scenario

You have been asked to design, implement, and test an addressing scheme for a company. The company has given you the network address that is suitable for the network, the topology, and the host requirements, as illustrated below and in the Lab4.pka file. You will implement and test your design.

# Instructions

You have been given the network address 192.168.110.0/24 by the company.

Open the Lab4.pka file. When the User Profile window appears, enter your full name and email address exactly as registered on Loop. Failure to provide this information will result in a 0 grade for this lab assignment.

# Requirements

## Host Requirements:

| LAN | Number of Addresses Required |
| --- | --- |
| LAN 1 – containing Host A | 58 hosts |
| LAN 2 – containing Host B | 29 hosts |
| LAN 3 – containing Host C | 13 hosts |
| LAN 4 – containing Host D | 6 hosts |

Design and implementation Requirements

- Create the addressing design.

- The subnets should be contiguous. There should be no unused address space between subnets.

- Provide the most efficient subnet possible for each LAN depending on the required number of hosts.

- Assign the subnets in the order of the number of hosts from the greatest to the least.

- Implement the designed scheme in Lab4.pka file. Assign the first host IP address in the subnet to the routers’ interface and the second host IP address to the host connected to that subnet

- For the point-to-point link between the routers, assign the first host IP address to R1’s interface and the second host IP address to R2’s interface.

- Build the routing tables for both routers.

- If the addressing design and implementation are correct, all hosts and devices should be reachable over the network.